Card Creation and Approval

A Southern California airframe manufacturer provides the outline below to create test cards:

STANDARD TEST PLANNING/REFERENCE DOCUMENTS

* "FTE" HANDBOOK

- ESTIMATED AERO PERF & S&C DATA

-- P.E. CURVES; AOA & BETA INDICATED-VS-TRUE

- THRUST SETTINGS

- SYSTEMS "LIMITATIONS"

- KNOWN USEFUL INFO: RUNWAYS, A/C DIMENSIONS/DWGS, ETC

* STRUCTURAL PLACARDS

- PREPARED BY STRUCTURAL DESIGN GROUP

- DISTILLATION OF STRUCTURAL DESIGN CRITERIA

PLUS

- DATA/CURVES HISTORICALLY KNOWN TO BE NEEDED

--OPERATION OUTSIDE "AFM" WEIGHTS & CG'S

-- FLOOR LOADINGS FOR BALLAST

-- ABNORMAL FUEL USAGE FOR CG CONTROL

-- LIMIT LOADS OF S/G'D PARTS 0

FLIGHT CARD CONTENT

* FLIGHT CARDS CONTAIN A SECTION TITLED "SYSTEM CONFIGURATION"

WHICH DETAILS

- THE SETUP OF AIRPLANE SYSTEMS RELEVANT TO THE TEST CARD

- THIS GENERALLY MEANS CALLING OUT THE REQUIRED POSITION OF

COCKPIT SWITCHES.

* A MATRIX IS DEVELOPED FROM THE PILOT'S EXPANDED CHECKLIST, &

REVIEW OF SPECIAL TEST SYSTEMS.

- TAKEOFF,

- CLIMB (BEGINNING AFTER GEAR, FLAP & SLAT RETRACTION),

- CRUISE (ABOVE FL180),

- APPROACH (BEGINNING BELOW FL180 & PRIOR TO GEAR DOWN, FLAP

OR SLAT EXTENSION),

- LANDING (BEGINNING AFTER GEAR DOWN & LANDING FLAPS

SELECTED).

- ALL COCKPIT SWITCH POSITIONS ARE LISTED FOR TAKEOFF

--IF/WHEN THEY ARE CHANGED FOR EACH REGIME THAT IS

NOTED.

-- "BOXED" ITEMS ARE FLIGHT TEST UNIQUE SYSTEM SWITCHES.

--  BASIC CONTROL LEVERS/CONTROLS ARE NOT LISTED (I.E.,

WHEEL, FLAP/SLAT, LANDING GEAR, THROTTLES).

* FLIGHT CARDS STATE "NORMAL FOR TAKEOFF, CLIMB,

ETC. EXCEPT" &

- THEN LIST DEVIATIONS FROM THE MATRIX FOR

TAKEOFF, ETC. AS APPROPRIATE TO THE TEST

* TEST REQUIREMENTS ARE OFTEN DIFFERENT THAN THE

MATRIX "NORMAL" CONFIGURATION.

* WHERE TEST RESULTS ARE HIGHLY DEPENDENT ON

CERTAIN SYSTEM CONFIGURATIONS THE FLIGHT CARD

STATES

- "NORMAL FOR TAKEOFF, CLIMB, ETC. WITH" &

- THEN LISTS ITEMS IN THE MATRIX TO EMPHASIZE

THEIR IMPORTANCE & PROVIDE A LAST SECOND

COCKPIT CHECKLIST.

A flowchart summary looks like:

-Prepare Preliminary Test Card

-Discussion with R & A Engineer

-OHA

-IFRs/ADRs

-Discussion with P.I.C.

-Structural Placards Document

-FTE Handbook

-Test Planning Meetings

|

|

- Test Conductors Finalize Individual Cards

|

|

- Test Conductors:Assemble Individual Cards into Deck for Each Flight and Review Test Aircraft Configuration Report/Records for Impact on Tests

|

|

- Preflight Meeting Chaired by Test Coordinator and Conducted Per Checklist

|

|

-Test Conductor Makes Agreed To Changes During Meeting

|

|

- Chief Experimental Test Pilot assigns OHA Category tests

|

|

- Approval signed by Test Conductor, Test Coordinator, P.I.C. and Chief Experimental Test Pilot (or designee)

An example of this company's test card is shown below:

MATRIX RUN #_______	 TEST RUN________	CARD __________

TEST:  LANDING PERFORMANCE AIR/GND DISTANCE:

MMEL CASE: 1 HYD SYS INOP IN ONE WHEEL INOP

TEST NO. 04-510.00, 04-520.09 	FUSELAGE NO.________  FLIGHT NO.________

TEST REFERENCE: FAR 25.125c, 25.1309,

TEST PURPOSE: Demonstrate landing air/ground distance performance with 6002 brakes, -306 A/S & -3 wheel speed transducers for the MMEL case of 1 HYD SYS INOP IN ONE WHEEL INOP.

TEST CONDITIONS	SYSTEM CONFIGURATION	NORMAL FOR LANDING 									WITH :

ALTITUDE : APPR			A/P : ON/OFF		ANTI-SKID:ARM

AIRSPEED : 1.3VS			ATS : ON/:F			APS : DISARM

FLAP/SLAT: /LND			# WHEEL-LH / RH		SPOILERS : ARMED

THRUST : NOTED (3 o FPA)	BRAKE SYS INOP		CAPT A/S : PROD

LDG GEAR : DN

WEIGHT/CG: HVY/FWD LIM	HVY = WT < MLW/FWD LIM = -7.5 to-5.1%+0.5-0.0%

INSTRUMENTATION DATA RATE: 400 SPS

OTHER: WIND KIT, RADIO CAR, PHOTO, FANS

NOTE: 3 GOOD RUNS REQUIRED @ EACH LANDING FLAP ---> 6 RUNS TOTAL

PROCEDURE

1.During final approach: - Confirm Arresting Gear Removed

(a) Verify brake temps are less than ______

(b) Complete FUEL/WEIGHT log entry

(c) Verify FTDS sample rate ---> set

2. Conduct a stabilized approach and landing at the following target values;

EPR:______ 		V/S:_____ FT/SEC / ______FT/MIN

1.3VS(+/-2): ____ KTS 	THROTTLE RETARD HT: _____ FT

- Monitor Airspeed: VREF +/-2 KTS from 500 - 50 ft AGL

- Minimize Thrust Changes Below 200 ft, EPR at or Above Target

- Monitor Winds/Air Quality & T/D Pitch Attitude

- Target T/D Sink Rate; 2 - 6 FPS, Do Not Exceed 8 FPS for Data Quality

3.  Approximately 1.0 seconds after MLG T/D, apply STEP, FULL PEDAL BRAKING to a FULL STOP.

Verify/backup auto-spoiler deployment. DO NOT USE REVERSE THRUST.

NOTE: USE ELEVATOR TO CONTROL DE-ROTATION AS REQUIRED max NLG SINK RATE = 10 FPS).

4 After coming to a full stop: (a) Complete FUEL/WEIGHT log entry.

(b) Verify INS TARES have been obtained.

(c) Inspect Gear, Tires, Brakes & Dampers As Req'd.

5. FOR BRAKE COOLING: Perform STOP & GO  OR Taxi clear of RWY, chock nose wheel (DO NOT USE PARKING BRAKE), advise ground crew reference cooling fans. Monitor brake temps.

STOP & CO TAKEOFF DATA: GW:	FLAPS: /	FE:	' OAT:

V1: 		VR:		V2:

TOFL:	'2nd SEG: %

- MAX BRAKE TEMP ALLOWABLE FOR TAKEOFF =

- EST BRAKE TEMP RISE FOR LDG PERF RUN =